- Download the Lisp routine from share point.

- Use command APPLOAD to load the lisp routine

- Make sure the file successfully uploaded. This is in the bottom left corner of the window.

- Type in the command ALIGNTOCENTER

- Select the first EOP

- Select the second EOP

- Select the Centerline

- The Centerline should now be centered between EOPs, at every vertex

- In some spots the Centerline may not seem “smooth” because there aren’t enough vertexes. In that case you can just add more vertexes and run the routine again

- Make sure to adjust the endpoints of the centerline

- If the EOPs are part of the same polyline, you will have to temporarily break the line by using the BREAKATPOINT command